Supplementary Table — CADENCE Alternans Metrics

Voltage and calcium alternans and substrate metrics from the 3-D optical-mapping pipeline, with output field, units, definition, and significance.

Conventions: maps are [R × C] grids masked to tissue (NaN outside). ALT = mean(odd − even) over beat pairs; ratio = |ALT| / mean(metric), dimensionless. Phase: +1 odd beats larger/longer, −1 even. Defaults: ratio > 0.10 present (> 0.20 strong), p < 0.05. Same-kind ratio maps may share a colour scale; duration and amplitude maps must not.

Voltage — Action-Potential Alternans

analyzeAPAlternans.m (3-D mode)

| Metric | Field(s) | Units | Definition | Significance | Threshold / flag |
| --- | --- | --- | --- | --- | --- |
| APD alternans | APD30_alt, APD50_alt, APD80_alt | ms | Beat-pair difference in APD at 30/50/80 % repolarisation. | Canonical repolarisation alternans and primary dispersion substrate. APD80 standard; auto-falls back to APD50/30 when unreachable at fast pacing. | ratio > 0.10 |
| APD alternans ratio | APD50_ratio_map | — | ¦APD alternans¦ / mean APD per pixel. | Magnitude normalised to local APD; feeds composite risk (saturates at 0.20). | >0.10 present; >0.20 strong |
| AP amplitude alternans | amp_alt_map, amp_ratio_map | a.u. / — | Beat-pair difference in AP amplitude (Vmax − Vrest). | Full-tissue reference map. Less specific — sensitive to signal / mechanical artefact. | interpret with SNR |
| Upstroke (dV/dt) alternans | dvdt_alt_map, dvdt_ratio_map | a.u./ms | Beat-pair difference in max upstroke velocity. | Excitability / conduction alternans; electrical correlate of the conduction panel; driver of 2:1 block. | — |
| Triangulation alternans | tri_alt_map | ms | Beat-pair difference in triangulation (APD90 − APD30). | Repolarisation-morphology instability; proarrhythmic (reduced repolarisation reserve). | — |
| Alternans phase | phase_map | +1 / −1 / 0 | Sign of APD alternans (+1 odd longer, −1 even longer). | Building block of concordance/discordance; anti-phase regions = discordant alternans. | [−1, 1] scale |
| Continuous phase angle | phase_angle_map | rad (−π…π) | Angle of FFT coefficient at 0.5 cyc/beat. | Spatial gradients reveal travelling alternans waves; pixels ~π apart are in anti-phase. | — |
| Spectral alternans index | spectral_map | power | Power at 0.5 cyc/beat in per-beat APD series. | Frequency-domain magnitude, robust to missed/extra beats. Needs ≥4 beats. | — |
| Statistical significance | pval_map, tstat_map | p / t | Pixel-wise paired t-test, odd vs even APD. | Separates sustained alternans from noise; gate low-SNR pixels. Needs ≥3 pairs. | p < 0.05 |
| AP load / release alternans | AP_release_alt_map, AP_load_alt_map, AP_amp_phase_map, AP_L_vamp_map, AP_S_vamp_map | — / a.u. | Wang 2014 applied to AP amplitude: release = 1 − S/L; load = D/L; L/S = large/small-beat amplitude. | Parallels the calcium load/release decomposition for AP-vs-Ca mechanism comparison. | release ∈ [0,1] |

Calcium — Ca²⁺-Transient Alternans

analyzeCaTransientAlternans.m (3-D mode)

| Metric | Field(s) | Units | Definition | Significance | Threshold / flag |
| --- | --- | --- | --- | --- | --- |
| CaT amplitude alternans | amp_alt_map, amp_ratio_map | a.u. / — | Beat-pair difference in Ca-transient amplitude (peak − onset), and its ratio. | Canonical calcium alternans and most-reported metric — SR Ca²⁺ cycling instability. Pairs with APD alternans. | ratio > 0.10 |
| CaT duration alternans | D50_alt_map, D80_alt_map, D50_ratio_map, D80_ratio_map | ms / — | Beat-pair difference in CaTD to 50 % / 80 % decay. | Calcium analogue of APD alternans. Reachability caveat: at fast pacing the transient may not decay to 80 % within the beat window — check level_coverage before trusting D80. | see level_coverage |
| Time-to-peak alternans | TTP_alt_map, TTP_ratio_map | ms / — | Beat-pair difference in onset-to-peak time. | Release-kinetics alternans — variability in SR Ca²⁺ release speed. | — |
| SR Ca release alternans | release_alt_map | — [0,1] | Wang 2014: 1 − S/L (L/S = mean large/small-beat amplitude, per-pixel). | Fractional-release instability — the release-driven component. Per-pixel L/S keeps it valid under discordant alternans. | 0 = none; →1 strong |
| SR Ca load alternans | load_alt_map | — (≥0) | Wang 2014: D/L, D = mean ¦diastolicL − diastolicS¦. | Diastolic-load instability. load ≈ 0 with release > 0 ⇒ release-driven alternans (common arrhythmogenic route). | ≈0 ⇒ release-driven |
| Large / small beat amplitude | L_mean_map, S_mean_map | a.u. | Per-pixel mean amplitude of locally larger (L) / smaller (S) beats. | Underlie release/load ratios; L ≥ S everywhere by construction. | — |
| Ca alternans phase | phase_map, phase_angle_map | +1/−1/0 · rad | Sign and continuous angle of Ca amplitude alternans. | Multiply by AP phase_map for Ca–Vm concordance: product +1 in-phase, −1 out-of-phase. | [−1, 1] scale |
| Spectral & significance | spectral_map, pval_map, tstat_map | power · p/t | Power at 0.5 cyc/beat; paired t-test odd vs even amplitude. | Same roles as the voltage versions. | p < 0.05 |
| CaTD reachability | level_coverage | fraction | Fraction of tissue-beats with a finite CaTD per decay level (diagnostic). | Data-quality gate: <0.5 at D80 means that map is mostly NaN — do not over-interpret. | warn if < 0.50 |

Substrate & Ca–AP Coupling

assess_arrhythmia_substrate.m

| Metric | Field(s) | Units | Definition | Significance | Threshold / flag |
| --- | --- | --- | --- | --- | --- |
| Concordance / discordance | concordance_ratio, is_discordant | fraction · bool | Fraction of significant pixels sharing the majority phase. | Discordant alternans steepens repolarisation gradients — classic precursor to block and reentry. | discordant if < 0.80 |
| Nodal lines | nodal_lines | logical | Pixels bordering both +phase and −phase regions. | Phase-reversal boundaries — highest-risk sites where unidirectional block initiates. | binary mask |
| Repolarisation dispersion | apd_disp_global, gradient_mag, max_gradient | ms · ms/px | SD of mean-APD map and its Sobel gradient magnitude/peak. | Large dispersion / steep gradients mark repolarisation borders — block lines and reentry substrate. | — |
| Ca–AP coupling | coupling_map, coupling_fraction, inphase_mask | r ∈ [−1,1] | Per-pixel Pearson r between per-beat Ca-amplitude and APD series. | r > 0 in-phase (Ca drives AP via NCX / ICaL); r < 0 out-of-phase (voltage drives Ca). Out-of-phase = higher-risk substrate. | >0.6 Ca-driven; <0.4 V-driven |
| Ca metrics (comparison) | ca_phase_map, ca_phase_angle_map, ca_alt_ratio_map | +1/−1/0 · rad · — | Calcium phase and alternans-ratio maps wired into the substrate struct. | Overlay Ca and AP phase/magnitude on the same significant-pixel footprint. | — |
| Diastolic Ca elevation | diast_ca_map, diast_ca_alt_map | norm. a.u. | Mean normalised diastolic Ca (last 10 % of interval) and its odd-even alternans. | Elevated diastolic Ca → DADs / triggered activity (independent risk axis); its alternans indicates SR-loading instability. | requires CaData |
| APD restitution slope | restitution_map, slope_gt1_mask, mean_slope | dAPD/dDI | Local slope of APD vs diastolic interval from odd/even (APD, DI) pairs. | Restitution-hypothesis instability predictor: slope > 1 dynamically unstable, alternans/VF-susceptible. | unstable if > 1 |
| Composite arrhythmia risk | risk_map, risk_global, risk_components | [0, 1] | Weighted sum (norm. to 1): alternans ratio (1.0) + APD gradient (1.0) + nodal proximity (1.0) + restitution>1 (1.0) + Ca out-of-phase (0.5) + diastolic Ca (0.5). | Single per-pixel susceptibility score integrating dispersion, dynamic instability, and Ca-driven amplification. | higher = higher risk |

Note: 3-D maps operate on optical fluorescence (F/F₀-style), so AP amplitude is in arbitrary units (a.u.), not mV. Significance cut-offs (ratio > 0.10, p < 0.05) are pipeline defaults; adjust to match the analysis run.